OSCILLOSCOPE - LE CROY WAVERUNNER LT374

Main Features:

Bandwidth: 500 MHz

Channels: Four

Sample Rate/ch: 2 GS/s

Maximum Sample Rate: 4 GS/s

Acq. Memory per Ch/Max: 250 kpt/500 kpt

LT374.pdf

ELECTRONIC LOAD – UNIWATT EL100

Main Features:

50V 2A 100W max load

POWER SUPPLY  - Hewlett Packard HP E3631A triple-output

Main Features:

Equipped GPIB and RS-232 as standard.

All programming is done to use SCPI (Standard Commands for Programmable Instruments).

80-watt triple output supply

Three independent 0 to +6V/5A and 0 to +/-25V/1A outputs.

The 6V output is electrically isolated from the +/-25V supply

The +/-25V outputs can be set to track each other.

HP E3631A.pdf

AC POWER SUPPLY - VAR630

Main Features:

Nominal voltage: 220Vac monophase, frequency: 50Hz;
Output voltage: 10-270 Vac;
Power output: 800VA;
Max output current: 3,5 A (7.5A, 15A, 20A);
Programmable ext reference output voltage: 0-10Vdc;
Tollerance: Vout +/-5% vs. reference output voltage 0-10Vdc;
Overvoltage protection programmable range 10÷270Vac;
Automatic and manual mode.

POWER SUPPLY - Thurlby Thandar EL302D

Main Features:

Dual outputs, 30V/2A

Independent digital voltage and current meters

for each output with 3 digit resolution

Constant voltage or constant current operation

DC output switch, automatic mode indication

TTI EL302D.pdf

ELECTRONIC LOAD – UNIWATT EL100

Main Features:

30V 100A 1kW max load

POWER SUPPLY - Elind 50HS6,4

Main Features:

Output voltage: 0-50V

Output current: 0-6.4A

Voltage Insulating (input-gnd, input-output, output-gnd) >50Mohm@500Vcc

Remote sensing for each conductor 0.5V

Line regulation: CV 0,01% + 0.005%; CC 0.02% + 0.01%

Load regulation: CV 0,01% + 0.005%; CC 0.02% + 0.01%

Residual ripple (20Hz-20.kHz): CV 300uV + 5uV/V; CC600uA + 50uA/A

PULSE PATTERN GENERATOR – Hewlett Packard HP8110A

Main Features:

150MHz

Variable pulse parameters
PLL module& Deskew module
Number of Channels: 2 ch

Maximum Frequency: 150 MHz

Frequency Accuracy: 5 %

Minimum Pulse Width: 3.3 ns

Maximum Pulse Width: 998 ms

Maximum Output Voltage: 20 V

Minimum Period: 6.65 ns

Maximum Period: 999 ms

Minimum Delay: 0.00 ns

Maximum Delay: 998 ms

Output Data Format: NRZ

Output Impedance: 50 Ohm

HP8110A.pdf

WIDEBAND POWER AMPLIFIER - Krohn-Hite 7500

Main Features:

75 Watts

Frequency Range: DC to 1MHz

Frequency Response: <±0.1dB

Distortion: <0.1%

Maximum Voltage: 140Vrms, Open Circuit

Voltage Gain: 0dB to 40dB

Variable DC Offset: 0V to 200V Peak

Short-Circuit Protection

KrohneHite7500.pdf

MULTIMETER - Hewlett Packard HP3478A

HP3478A.pdf

TESLAMETER

Range selection: 0.2T, 2T

LOGIC PROBE - HP LogicDart E2310A

Main Features:

100 MSa/s logic timing for TTL / CMOS/ ECL / Custom

3 channel timing display with tristate

Logic monitor with 15 ns glitch detect

3 1 / 2 digit dc voltage to 35 volts

Continuity, frequency, resistance, & diode test

HP2310A.pdf

MULTIMETER – BBC M2037

Main Features:

Vcc range:300mV-1kV

Vac range:300mV-1kV

Acc range:300uA-3A

Aac range:300uA-3A

Ohm range: 300ohm-30Mohm

BBC M2037.pdf

OSCILLOSCOPE MULTIMETER HANDHELD  - TEKTRONIX THS720A

Main Features:

Bandwidth100 MHz

Sample Rate 500 MS/s (Each Channel)

Time/Division Range5 ns to 50s/div

Channels - Two.

THS720.pdf

THERMOMETER DATA-LOGGER – Delta Ohm DO9416

Main Features:

Potable microprocessor instrument for thermocoupler probes type K, J, T, E, R, S o B

Principal characteristics:
double liquid crystal display,12.5 mm high
double input
optoinsulated RS232C serial output,baud rate selectable from 300 to 19200 baud

storage of the maximun,minimum and mean temperature values
relative measurements            
power supply : 9v battery (50 hours' use)
measuring range:

K 200°C.... + 1370°C 328°F + 1999°F

J 1 00°'C.... + 750°C 148°F + 1382°F

T 200°C . 450°C 328°F + 662°F

E 200°C . 750°C 328°F + 1382°F

R +200°C.... + 1480°C +392°F.... + 1999°F

S +200°C + 1480°C +392°F . 1 999°F

B +200°C . 1800°C +392°F + 1999°F

Resolution: 0,1°C/0.1°F 
from -199.9 to 199.9  1°C/1°F beyond that

Precision (instrument alone) with temperature from 18°C to 25°C:

DO9416.pdf

DO9416 manual.pdf

THERMOMETER DATA-LOGGER – Delta Ohm DO9417

Main Features:

Potable microprocessor instrument for Pt100 probes

Principal characteristics:
double liquid crystal display,12.5 mm high
double input
optoinsulated RS232C serial output,baud rate selectable from 300 to 19200 baud

storage of the maximun,minimum and mean temperature values
relative measurements            
power supply : 9v battery (50 hours' use)
measuring range:-200°C ...+800°C

resolution: 0.1°C from -199.9°C to +199.9°C, 1°C beyond

DO9417.pdf

DO9417 manual.pdf

SCOPEMETER - Fluke 105B ScopeMeter Series II

Main Features:

2 channel and ext trigger - video trigger.
100MHz bandwidth, 5Gs/s sample rate.
Normal true RMS 3000 count DMM plus db.
Complete with software

Fluke105B.pdf

TRUE ROOT-MEAN-SQUARE VOLTMETER - Hewlett-Packard 3400A

Main Features:

Voltage range 1mV to 300V full scale

Frequency range 10 Hz to 10 MHz

Maximum DC input 600V

Response time less than 5 sec to a step function

Input impedance 10 MOhm and less than 50 pF for 1mV to 0.3V range

10 MOhm and less than 20 pF for 1V to 300 V range

Meter accuracy

10Hz-50Hz 5%

50Hz-1MHz 1%

1MHz-2MHz 2%

2MHz-3MHz 3%

3MHz-10MHz 5%

AC to DC Converter accuracy

10Hz-50Hz 5%

50Hz-1MHz 0.75%

1MHz-2MHz 2%

2MHz-3MHz 3%

3MHz-10MHz 5%

OSCILLOSCOPE - Hewlett Packard HP54520A

500 MHz, 2 Ch. 1 GS/s

Internal 3-1/2" floppy drive and HP-IB.

Main Features:

Bandwidth 500 MHz

Number of Channels 2 ch

Simultaneous Channels 2 ch

Simultaneous Maximum Sampling Rate/ch 500 MSa/s

One ch. only max. sampling rate 1 GSa/s

Max. Single Shot bandwidth 250 MHz

Max. Record Length 501 pt/sec

Min. Vertical Sensitivity 1 mV/div

Maximum Vertical Sensitivity 5 V/div

Rise time 700 ps

Number of Bits 8 bits

Input Impedance 1 MOhm

Input Impedance (alternate) 50 Ohm

Input Coupling AC,DC

Maximum Input Voltage 250 Vrms

Main time base - lowest 500 ps/div

Main time base - highest 5 s/div

Timebase accuracy 0.005 %

Trigger Source External,Internal

Trigger Modes Auto,Edge,Glitch,Logic,Trigger,TV Trigger

Sensitivity 0.5 mV

Minimum Glitch Trigger 1 ns

Display Type CRT Monochrome

Display modes average, envelope, persistence

NETWORK/SPECTRUM ANALYZER – Hewlett Packard HP 4396A 1.8GHz

Vector network analyzer specifications:

Frequency operations: 100kHz to 1.8GHz with 1mHz resolution

Integrated synthesized level source: -60 to +20dBm of output power with 0.1dB resolution

Dynamic magnitude: ±0.05 dB

Phase accuracy ±0.3°

Spectrum analyzer specifications: the 3496A

Frequency operations: 2Hz to 1.8GHz with resolution-bandwidths (RBWs) spanning 1Hz to 3MHz Direct A/D conversion (no LOG amplifier is used) results in ±1.0dB overall level accuracy.

Noise sidebands fall below -105dBc/Hz offset 10kHz from carriers below 1GHz

Sensitivity: -150dBm/Hz @ 10MHz and -147dBm @ 1GHz.

Color CRT and built-in disk drive/RAM disk

HP4396A.pdf

DIGITAL OSCILLOSCOPE - LeCroy 9374

Main Features:

Bandwidth 1 GHz

Number of Channels 4 ch

Simultaneous Maximum Sampling Rate/ch 500 MSa/s

One ch. only max. sampling rate 2 GSa/s

LECROY9354.pdf

FUNCTION ARBITRARY WAVEFORM GENERATOR – Agilent 33250A

Specifications:

Sine, square, pulse, ramp, noise

AM, FM, FSK modulation

80 MHz sine and square wave outputs

50 MHz pulse waveforms with variable rise/fall times

12-bit, 200 MSa/s, 64K-point deep arbitrary waveform

GPIB and RS-232C interface

33250A.pdf

DIGITIZING OSCILLOSCOPE SYSTEM– Hewlett Packard HP54720A

Main Features:

FFT, measurement limit test, color graded display, histograms

3½" MS-DOS compatible disk drive

Time base scale (full screen  is 10 div): 100 ps/div to 1 s/div

Maximum sampling rate real time: 4 GSa/s
Equivalent time: 500 MSa/s

Maximum waveform record length/plug-in: 32,768 points

PLUG-INs:

HP 54711A Attenuator

50ohm attenuator plug-in with external trigger. (1.5GHz bandwidth with HP 54710A and 54720A.

HP 54713B Amplifier

50ohm /1Mohm amplifier with internal trigger. (500MHz bandwidth with HP54710A and 54720A).

HP 54721A Two-Wide Amplifier Plug-In

Bandwidth: 1GHz with HP 54710A and 54720A.

Sample rate 4GSa/s.

DIGITIZING OSCILLOSCOPE SYSTEM – Tektronix TDS 510A

Main Features:

500 MHz Bandwidth

Sample Rates to 500 MS/s

4 Channels

Width, 2 ns Glitch, Runt, State, and Pattern Triggering

1 mV to 10 V/div Sensitivity

Infinite and Variable Persistence

8-Bit Vertical Resolution Up to 12-Bit in Hi-Res Mode

1% Vertical Accuracy

FFT, Integration, Differentiation (option 2F)

25 Automatic Measurements

GPIB Programmable

3.5 in. Floppy Disk Drive

TDS510A.pdf

HIGH VOLTAGE POWER SUPPLY – FUG HCN 35-6 500

Features:

Output voltage: 0-6,5kV

Output current: 0-5 mA

FUG.pdf

LCR METER - Hewlett Packard HP4263A

Main Features:

0.1% basic accuracy

100 Hz, 120 Hz, 1 kHz, 10 kHz, 100 kHz test frequencies

50 m, 100 m, 250 m, 500 m, 1 Vrms test levels

High-speed measurement: 25 ms

High-speed contact check

Quick test recovery

Wide capacitance test range

Front-end protection

Built-in comparator

Transformer parameter measurements

HP4263A.pdf

DIGITAL OSCILLOSCOPE – LECROY 9370

Main Features:

1 GHz Bandwidth, 2 GS/s

Up to 8M Point record length

8-bit vertical resolution, 11 with ERES option

Two Channel

Portable Hard Disk (PCMCIA III), Memory Card and DOS Compatible Floppy Disk options

Peak Detect

Glitch, Pattern, Qualified, Interval, Dropout and TV Triggers

Fully programmable via GPIB and RS-232-C

Internal Graphics Printer Option

Automatic PASS/FAIL testing

Advanced Signal Processing

LECROY9370.pdf

DIGITAL OSCILLOSCOPE – Agilent/HP54845A

Features:

1.5 GHz bandwidth

2 or 4 channels

8 GSa/s sample rates (2 channel mode)

4 GSa/s sample rates (4 channel mode)

Simple, analog-like front panel

Windows® 98-based graphical user interface

USB Test option with embedded MatLab

Time correlation between 16700 logic analyzer and scope

Built-in 120 MB floppy disk

LAN printing and file sharing

Eye diagrams and mask testing

Communications mask test kit option

Voice Control option for hands free operation

Web-enabled remote control from any web browser

Email on trigger

HP54845A.pdf

HP54845Ab.pdf

HIGH SPEED PULSE GENERATOR - Hewlett Packard HP8131A

Features:

Timing period: 2 ns to 99.9 ms

Delay: 20 ns to 99.9 ms

Pulse width: 500 ps to 99.9 ms

Double pulse: 2 ns to 99.9 ms

Transition times: <200 ps

Output amplitude: -4.90 V to +5.00 V high level, -5.00 V to +4.90 V low level

Output accuracy: ±3% amplitude

Output resolution: 10 mV

Polarity: pos and neg outputs

HP-IB interface

Output connectors: BNC(f)

100-120/220-240 Vac ±10%; 48-66 Hz operation

Weight: 45 lb; Size: 145 mm (5.7 in)H x 426 mm (16.75 in)W x 525 mm (20.65 in)D

HP8131A.pdf

DIGITAL OSCILLOSCOPE – LECROY 9374

Main Features:

1 GHz Bandwidth, 2 GS/s

Up to 8M Point record length

8-bit vertical resolution, 11 with ERES option

Two Channel

Portable Hard Disk (PCMCIA III), Memory Card and DOS Compatible Floppy Disk options

Peak Detect

Glitch, Pattern, Qualified, Interval, Dropout and TV Triggers

Fully programmable via GPIB and RS-232-C

Internal Graphics Printer Option

Automatic PASS/FAIL testing

Advanced Signal Processing

LECROY9370.pdf

WAVEFORM GENERATOR - Hewlett Packard HP33120A

Main Features:

15 MHz sine and square waves

Sine, triangle, square, ramp, noise, sin(x)/x, cardiac & more

Arbitrary waveforms 12-bit, 40 MSa/s, 16k deep

Direct digital synthesis for 10 µHz or 10-digit resolution

50 mV to 10 V p-p output into 50 Ohms with 3-digit resolution

Linear and log sweeps

AM, FM, FSK, and Burst modulation

GPIB (SCPI) and RS-232 included

HP33120A.pdf

LOGIC ANALYSIS SYSTEM - Hewlett Packard HP16500B

Main Features:

###### Logic Analysis System Mainframe 16500B

Systems configured to exact requirement

Up to 1000 channels of 100 MHz state OR

Up to 640 channels of 100 MHz timing with 1Mb/channel depth

All data is time correlated

Built-in hard drive with software preloaded

Touch screen, keyboard & mouse user interfaces

Available Modules:

16520A 50MSa/s pattern generator

16522A 200Mvector/s pattern generator

16532A 2 Channel 1GSa/s Oscilloscope Acquisition Card

16550A 102 Channel 100 MHz State/500 MHz Timing Card

16500B user guide.pdf

16500B user references.pdf

16500B settings up.pdf

16500B programmer guide.PDF

16500B upgrades.PDF

PRECISION CALIBRATION SOURCE - DIGISTANT Model 4405

Main Features:

Micro-processor controlled current voltage source

Voltage range:10 µV to 11.000 V

Current range:2 µA to 22.00 mA

NC-battery with overload and low-discharge protection

BURSTER4405.pdf

PORTABLE THERMOMETER-HYGROMETER DATA-LOGGER – Delta Ohm DO 9406

Instrument measurements:

% U.R. of relative humidity
D.P. dew point
g/m3 Quantity of water per m3 of air with reference to 0°C and a pressure of 1013 mbar
°C, °F Temperature in °C or °F
REL Relative measurements
MAX maximum value over a given period
MIN minimum value over a given period
MED mean value over a given period.

DO9406.pdf

DO9406 manual.pdf

ANALOG OSCILLOSCOPE – Tektronix 2246

Main Features:

100 MHz 4 Independant Channels

Dual 2 nS/div timebase

Delayed Sweep

2 mV/div Vertical Sensitivity

On screen cursor & readout for measurement of Volts & Time

ANALOG OSCILLOSCOPE – Tektronix 2245

Main Features:

150MHz bandwidth at probe tip

1 ns/div sweep rate

2 mV/div vertical sensitivity

250MHz minimum triggering bandwidth

Four independent channels

ELECTROMETER - KEITHLEY 6512

KEITHLEY6512.PDF

UNIVERSAL COUNTER – Hewlett Packard 5315A

Main Features:

Measurements range: 0.1Hz to 100MHz range.

Measurement Period: 10 ns to 105 Sec
Time Interval: 100 ns to 105 Sec
Time Interval Average: 0 ns to 105 Sec
Time Interval Delay: holdoff, variable delay of nominally 500 µs to 20 ms between START (channel A) and STOP (channel B)
And Ratio: 0.1Hz to 100MHz, both channels.

Agilent 5315A.pdf

NETWORK/SPECTRUM ANALYZER - Anritsu MS420K

Main Features:

Frequency Range: 10Hz to 30MHz.  Resolution: 0.01Hz

Input Channel: 2 channels (R and T) 50 ohm impedance

Wide measurable level range of over 150dB, allowing the measured device to be checked at actual operational levels

High-performance synthesizer to enable high resolution measurements

High-speed measurement of 2ms/point useful for  mass production of parts.

High-precision group delay measurements

Logarithmic frequency sweep

Level sweep for non-linearity tests; variable input levels make the instrument suitable for characteristics tests

Built-in GP-IB interface for remote control of front-panel functions

Level calibration by using the output of the test-signal source

External MG440A Synthesizer or MG443B SLG can be connected to check frequency response of conversion loss at different input/output frequencies

PULSE/FUNCTION GENERATOR - Hewlett Packard 8111A

Main Features:

Sine, triangle, square, and haverfunctions

20 MHz, 32 V p-p for all waveforms

Variable duty cycle or pulse width

Trigger, gate, VCO, and optional burst

Digital display for all parameters

Error recognition

hp8111a.pdf